WRO Future Engineers Robot Documentation

1. Introduction

This project presents the development of an autonomous robot designed for the WRO Future Engineers category. The robot is built as a compact self-driving car prototype based on LEGO Education SPIKE Prime. The main objective of the project is to create a vehicle that can move autonomously, maintain stable motion, detect important field colors from short distance, and make directional decisions in a controlled and repeatable manner.

From the beginning of the project, the design priority was defined as stability over speed. Rather than optimizing the robot for aggressive motion, the engineering focus was placed on consistent steering, reliable propulsion, short-range color detection, and predictable autonomous behavior. This decision was made because in competition conditions a stable robot is more effective than a fast but inconsistent one.

The robot is designed according to the car-like concept required by the category. It uses a four-wheel layout, one motor for propulsion, and a separate motor for steering. This approach matches the engineering logic of a real autonomous vehicle and supports compliance with the competition rules.

2. Design Objective

The design objective of the robot is to achieve the following:

stable forward movement,

reliable steering control,

short-range field color recognition,

autonomous direction selection,

rule-based reaction to track conditions,

and reproducible engineering behavior.

The project does not attempt to maximize speed at the early stage. Instead, it focuses on building a robust autonomous platform that can later be optimized once the basic motion and sensing systems are proven reliable.

3. Mechanical Design

3.1 Overall concept

The robot uses a four-wheel car-style chassis. This concept was selected to reflect the category’s self-driving car philosophy and to avoid differential-drive motion. The drive system is based on a single propulsion motor, while the steering is handled by a separate steering motor. This is important because the rules do not permit one motor per side as a differential-drive solution; instead, the drive wheels must be mechanically connected.

3.2 Chassis structure

The chassis was designed to be:

compact,

rigid,

balanced,

and easy to maintain.

The SPIKE Prime Hub is placed near the center of the robot in order to improve balance and reduce instability during turning. The frame was reinforced to minimize bending and vibration. This was important because flexible chassis structures can reduce steering precision and cause inconsistent motion.

3.3 Drive system

The drive system uses one motor connected to the drive axle. The drive wheels are mechanically linked, which is aligned with the competition requirement for physically connected propulsion.

The motor selection was made with emphasis on controllability. In early development, it became clear that high nominal speed alone does not guarantee good practical performance. Therefore, the final concept favors a more stable, torque-suitable drive behavior instead of maximum raw speed.

3.4 Steering system

The front wheels are controlled by a separate steering motor. This creates a true car-like steering model instead of skid steering. The steering mechanism was calibrated experimentally in order to determine:

center position,

left turning position,

right turning position,

and softer intermediate steering values.

The addition of soft steering values was an important engineering improvement because it allowed the robot to perform smaller corrections without excessive oscillation.

3.5 Stability considerations

A major design principle was that the robot should drive straight in a stable way even before advanced autonomy is added. For that reason, the alignment of wheels, steering geometry, chassis rigidity, and weight placement were treated as critical mechanical factors.

4. Electronics and Hardware Integration

4.1 Main controller

The main controller is the LEGO SPIKE Prime Hub, which is used for sensor reading, motor control, and decision execution. SPIKE Prime was selected because it offers a structured educational robotics platform with reliable integration between controller, motors, and sensors.

4.2 Motors

The robot uses:

one motor for propulsion,

one motor for steering.

This division of roles makes the robot easier to control and keeps the design aligned with the Future Engineers concept of an actual self-driving car.

4.3 Sensor system

The robot uses a SPIKE Prime color sensor for short-range color recognition. The sensor is mounted in the front lower part of the robot so that it can reliably detect colors when the robot is physically close to field markings or obstacles.

This sensor choice was intentional. The SPIKE color sensor is not intended for long-range vision; instead, it is best used for:

close-range color confirmation,

start-direction detection,

and near-object field recognition.

4.4 Start and power logic

The intended competition operating mode is:

the robot is powered on,

the program enters waiting mode,

the robot starts only after the start command.

This follows the required autonomous startup logic for the competition environment.

4.5 Sensor placement rationale

The color sensor was placed low and forward for the following reasons:

to reduce interference from ambient light,

to increase reliability of close-range color reading,

and to make the robot detect the target color only when it is within useful sensing distance.

This placement was tested as part of the engineering process rather than assumed in advance.

5. Software Architecture

5.1 General structure

The software was designed using a modular approach. The main logic is divided into separate functional blocks so that each subsystem can be tested independently and then integrated into the complete robot.

The software architecture concept includes:

configuration and constants,

drive control,

steering control,

sensor reading,

decision logic,

and challenge-specific behavior.

5.2 Control philosophy

The software uses a stability-first control philosophy. This means:

no aggressive steering changes unless necessary,

no unnecessary speed increase,

no overcomplicated behavior in the early stage.

This approach supports repeatable movement and easier troubleshooting.

5.3 Basic control layers

The software can be understood in the following layers:

Movement layer

Controls forward motion and stopping.

Steering layer

Controls center, soft turn, and hard turn states.

Sensor layer

Reads color information and interprets the visible field state.

Decision layer

Converts sensor readings into high-level actions such as:

select clockwise direction,

select counterclockwise direction,

keep left,

keep right,

continue straight.

Execution layer

Applies the selected action to the motors.

5.4 Direction logic

For the Open Challenge, the software concept includes reading blue and orange field markers in order to determine the route direction. The intended behavior is:

if blue is detected first, the robot selects one direction,

if orange is detected first, the robot selects the opposite direction.

This strategy allows the robot to adapt to the field orientation before beginning full movement.

5.5 Obstacle logic

For obstacle-related behavior, the rule-based interpretation is:

red → keep right

green → keep left.

This means the software does not simply “avoid” red or green. Instead, it uses the detected color as a lane-side instruction, which is closer to the formal competition rule interpretation.

6. Open Challenge Strategy

The Open Challenge strategy is based on stable one-lap and later multi-lap execution.

6.1 Initial direction selection

At the beginning of the run, the robot uses color information to determine the direction of travel. This is based on field markers visible at the start.

6.2 Stable forward motion

After the direction is selected, the robot enters a stable driving mode. At this stage, the priority is:

to remain centered on the intended path,

to avoid strong oscillation,

and to complete the lap reliably.

6.3 Progressive development plan

The Open Challenge development is divided into stages:

stable forward drive,

direction detection,

one-lap consistency,

multiple-lap reliability.

This staged strategy reduces risk and allows improvements to be tested incrementally.

7. Obstacle Challenge Strategy

The Obstacle Challenge logic is based on short-range color detection.

7.1 Color-based decision

When the robot approaches a relevant field element, the color sensor confirms the visible color.

7.2 Rule-based maneuver

The robot interprets:

red as a command to keep right,

green as a command to keep left.

7.3 Stability principle

Instead of reacting violently to every reading, the robot is intended to:

confirm the color,

apply the appropriate steering behavior,

and return to stable forward motion.

This prevents unstable movement and improves repeatability.

8. Development Process

8.1 Concept stage

At the start of the project, the team defined the robot as a car-like autonomous platform rather than a differential-drive robot.

8.2 Mechanical prototyping

The first prototypes focused on:

chassis size,

wheel arrangement,

motor placement,

and steering linkage.

8.3 Steering calibration

After the robot could move, steering positions were tuned experimentally until the robot could drive straight and make controlled turns.

8.4 Sensor testing

The color sensor was then tested in different positions. Through experimentation, it was confirmed that close-range mounting gave the most reliable results.

8.5 Software integration

After separate tests of movement and sensing, the control logic was integrated step by step:

movement only,

steering only,

color reading,

direction logic,

and autonomous response.

8.6 Iterative improvement

Each test stage revealed small problems that required adjustment. These included:

steering overcorrection,

weak straight-line consistency,

unstable sensor timing,

and the need for softer control values.

The robot was therefore improved through iterative engineering rather than fixed in a single design pass.

9. Engineering Decisions and Trade-Offs

Several important decisions shaped the final concept.

9.1 Stability over speed

A fast robot is not useful if it cannot finish reliably. Therefore, stability was chosen as the main optimization target.

9.2 Car-like steering over differential steering

This decision was made both for realism and for rule compliance.

9.3 Short-range color sensing over long-range vision

The color sensor was used in the role where it is strongest: near-field detection. This reduced complexity and increased reliability.

9.4 Modular software development

Developing and testing subsystems separately made debugging easier and made the engineering process more transparent.

9.5 Simplicity in early autonomy

The first goal was not to build a fully complex autonomous system immediately. Instead, the team aimed to create a controllable and explainable robot first, and then expand its autonomy gradually.

10. Testing and Iteration

Testing was treated as a core part of the engineering process.

The following aspects were repeatedly tested:

straight driving,

turning accuracy,

reaction to color,

consistency of sensor reading,

and repeatability of behavior.

Each round of testing was used to improve:

steering values,

chassis alignment,

sensor position,

and control thresholds.

This iterative method made the robot more reliable and also provided evidence for documentation and justification of engineering decisions.

11. Reproducibility

The robot concept was developed in a way that supports reproducibility:

the mechanical structure is based on SPIKE Prime components,

the sensing strategy is clearly defined,

the software logic is modular,

and the engineering decisions are explicitly explained.

This is important because a good engineering project should not only work, but also be understandable, explainable, and reproducible by others.

12. Current Development Status

At the current stage, the robot has:

a stable car-like chassis,

one propulsion motor,

one steering motor,

a front-mounted color sensor,

and a software concept for direction-based autonomous driving.

The robot already demonstrates the essential engineering principles of the category:

mobility, steering, sensing, decision making, and iterative refinement.

Future work will focus on:

improving lap consistency,

increasing robustness,

and refining challenge-specific behaviors.

13. Conclusion

The robot was developed as an engineering system rather than as a simple assembly task. Its design required:

conceptual planning,

mechanical construction,

calibration,

sensor placement decisions,

software development,

and repeated testing.

The final concept reflects a clear engineering philosophy: a stable, rule-aligned, explainable autonomous vehicle based on SPIKE Prime. The project emphasizes sound mechanical design, controlled motion, and practical sensor-based decision making, which together form a strong foundation for continued development in the WRO Future Engineers category.